SmartGreenAI: Cilantro Crop

Technical Evidence — Sprint 1

IoT and Cloud Monitoring

Data acquisition prototype for environmental and soil moisture sensors

User stories covered: US-03, US-04, US-05

Date: _______________     Team: _______________

# 1. Purpose of this document

This document presents the technical evidence corresponding to Sprint 1 of the SmartGreenAI: Cilantro Crop project. Its purpose is to document the design, implementation and validation of the IoT layer data acquisition prototype, and to report on the status of user stories US-03, US-04 and US-05.

The scope of this sprint is limited to local measurement capture and its emission in a structured, traceable format. Cloud transmission and historical storage belong to later stages of the project.

# 2. Scope and execution conditions

During the execution of this sprint the team did not have physical sensors available. For this reason, development and validation were carried out in an electronic circuit simulation environment, producing measurements equivalent to those the real sensors would generate.

This decision is aligned with general task number 10 of the project document, which states that the system shall be validated using simulated and/or real sensor data. Consequently, validation by simulation constitutes an accepted method within the defined scope, and not a provisional substitute.

| Aspect | Status in Sprint 1 |
| --- | --- |
| Physical sensors | Not available during the sprint |
| Validation method | Electronic circuit simulation |
| Data capture | Implemented, validation partially pending |
| Traceable data format | Implemented and validated |
| Cloud transmission | Out of scope (Sprint 2) |
| Historical storage | Out of scope (Sprint 2) |

# 3. Prototype architecture

The prototype consists of an Arduino UNO microcontroller acting as the acquisition node, two soil moisture sensors and one temperature sensor. The microcontroller performs periodic readings of the three sensors and emits each measurement individually and identified.

## 3.1 Components used

| Component | Identifier | Function |
| --- | --- | --- |
| Arduino UNO | U1 | Data acquisition node |
| Soil Moisture Sensor | SEN1 / SM-01 | Soil moisture measurement, point 1 |
| Soil Moisture Sensor | SEN2 / SM-02 | Soil moisture measurement, point 2 |
| TMP36 temperature sensor | U4 / TMP-01 | Ambient temperature measurement |

## 3.2 Wiring diagram

The following table documents the pin assignment validated in the circuit schematic. Analog pins A4 and A5 remain free for the addition of further sensors in subsequent sprints.

| Arduino pin | Connection | Description |
| --- | --- | --- |
| A0 | VCC of SEN1 and SEN2 | Shared switched power supply for both soil moisture sensors |
| A1 | SIG of SEN1 | Analog signal of soil moisture sensor SM-01 |
| A2 | VOUT of U4 | Analog signal of temperature sensor TMP-01 |
| A3 | SIG of SEN2 | Analog signal of soil moisture sensor SM-02 |
| 5V | +VS of U4 | Power supply for the temperature sensor |
| GND | GND of SEN1, SEN2 and U4 | Common ground reference |
| A4, A5 | Unused | Reserved for future sensors |

Note on switched power: the soil moisture sensors are powered through pin A0, which is activated only during the instant of the reading and deactivated immediately afterwards. This practice reduces electrolytic corrosion of the electrodes, extending sensor service life in a real installation. Section 6.2 discusses a possible relationship between this approach and the anomaly detected during testing.

Space reserved for the circuit schematic and simulation environment screenshots:

# 4. Identification and traceability scheme

To satisfy user story US-05, a hierarchical identifier scheme was defined that allows every measurement to be traced back to its source sensor and cultivation area. This scheme constitutes the basis on which sensors will be registered in the database during Sprint 2.

| Field | Current value | Description |
| --- | --- | --- |
| greenhouse_id | GH-01 | Unique greenhouse identifier |
| area_id | AREA-1 | Cultivation area identifier within the greenhouse |
| sensor_id | SM-01 | Soil moisture sensor, measurement point 1 |
| sensor_id | SM-02 | Soil moisture sensor, measurement point 2 |
| sensor_id | TMP-01 | Ambient temperature sensor |

## 4.1 Measurement record structure

Each measurement is emitted as an independent record in JSON format, containing the fields required for later storage in the cloud:

| Field | Type | Description |
| --- | --- | --- |
| greenhouse_id | Text | Source greenhouse |
| area_id | Text | Source cultivation area |
| sensor_id | Text | Unique sensor identifier |
| type | Text | Measurement type: soil_moisture or temperature |
| value | Numeric | Measured value |
| unit | Text | Unit of measurement: raw or C |
| timestamp_ms | Numeric | Milliseconds elapsed since device start-up |

Note on the timestamp: the simulation environment does not provide a real-time clock, so the timestamp_ms field records the milliseconds elapsed since the microcontroller started. In the final implementation this value must be replaced by an absolute timestamp, obtained from an RTC module or assigned by the server upon reception.

# 5. Soil moisture calibration ranges

Soil moisture sensors deliver a raw analog value in the range of 0 to 1023. The following preliminary level classification was defined and used during prototype testing:

| Range (raw value) | Classification | Preliminary agronomic interpretation |
| --- | --- | --- |
| 0 – 199 | Very dry | Critical condition, irrigation required immediately |
| 200 – 399 | Dry | Below target range, irrigation recommended |
| 400 – 599 | Adequate | Within the target cultivation range |
| 600 – 799 | Moist | Above target range, irrigation not required |
| 800 – 1023 | Very moist | Risk of saturation, verify drainage |

These thresholds correspond to a preliminary calibration performed in the simulation environment. They must be adjusted experimentally once physical sensors and real growing substrate are available, since sensor response varies with soil type and installation conditions.

# 6. Acquisition test results

A continuous acquisition test was executed on the prototype in order to verify that each of the three sensors emitted its measurement independently and correctly identified. The two soil moisture sensors were deliberately set to opposite conditions: SM-01 at the dry end of the range and SM-02 at maximum moisture capacity. The following is a representative extract of the records obtained:

{"greenhouse_id":"GH-01","area_id":"AREA-1","sensor_id":"TMP-01",

"type":"temperature","value":24.78,"unit":"C","timestamp_ms":6392}

{"greenhouse_id":"GH-01","area_id":"AREA-1","sensor_id":"SM-01",

"type":"soil_moisture","value":0.00,"unit":"raw","timestamp_ms":6976}

{"greenhouse_id":"GH-01","area_id":"AREA-1","sensor_id":"SM-02",

"type":"soil_moisture","value":872.00,"unit":"raw","timestamp_ms":7118}

{"greenhouse_id":"GH-01","area_id":"AREA-1","sensor_id":"TMP-01",

"type":"temperature","value":24.78,"unit":"C","timestamp_ms":7257}

{"greenhouse_id":"GH-01","area_id":"AREA-1","sensor_id":"SM-01",

"type":"soil_moisture","value":0.00,"unit":"raw","timestamp_ms":7839}

{"greenhouse_id":"GH-01","area_id":"AREA-1","sensor_id":"SM-02",

"type":"soil_moisture","value":872.00,"unit":"raw","timestamp_ms":7983}

{"greenhouse_id":"GH-01","area_id":"AREA-1","sensor_id":"TMP-01",

"type":"temperature","value":24.78,"unit":"C","timestamp_ms":8121}

## 6.1 Validated aspects

The obtained record confirms the following points:

- The three sensors emit measurements independently and cyclically, without interference between them.

- Each measurement includes its unique sensor identifier, allowing the origin of every data point to be clearly distinguished.

- The greenhouse and area fields are correctly propagated to all measurements.

- The timestamp increments consistently between successive readings.

- The temperature sensor reports a value within the expected range for normal ambient conditions, confirming that the analog acquisition path is functional.

- SM-01 reports 0.00, consistent with the dry condition it was deliberately set to.

- SM-02 reports 872.00, consistent with the maximum-moisture condition it was deliberately set to, confirming that the two soil moisture sensors respond independently and across distinct portions of the measurement range.

## 6.2 Resolution of the previously detected anomaly

An earlier test run had shown both soil moisture sensors reporting an identical constant value of 0.00 despite being set to opposite conditions, which indicated a fault in the soil moisture acquisition path. That test was repeated under the same opposite-condition configuration, with the results shown above. SM-01 and SM-02 now report clearly differentiated values consistent with their configured conditions, confirming that the soil moisture acquisition path is functioning correctly and that the previously identified anomaly has been resolved.

This result also confirms, retroactively, that the identification scheme and record format were never the source of the earlier issue, as anticipated in the previous version of this document.

# 7. User story status

| Story | Description | Status |
| --- | --- | --- |
| US-03 | Collection of temperature and ambient humidity measurements | Partial |
| US-04 | Collection of soil moisture measurements | Completed |
| US-05 | Unique sensor identification and association to an area | Completed |

Justification for the partial status of US-03: the temperature measurement was implemented and validated satisfactorily. The ambient humidity measurement could not be implemented because the simulation environment used does not provide a component equivalent to the DHT11 or DHT22 sensors, and the team did not have physical sensors available during the sprint. This functionality is formally planned for incorporation once the corresponding hardware becomes available.

Justification for the completed status of US-04: the circuit, the acquisition code and the traceable output format for both soil moisture sensors were implemented and validated. An initial test revealed an anomaly in which both sensors reported identical values; the test was repeated under an opposite-condition configuration (section 6.2) and produced correctly differentiated readings for both sensors, confirming that soil moisture data is being collected as intended.

Justification for the completed status of US-05: every measurement is emitted with a unique sensor identifier and associated with a greenhouse and cultivation area. This behaviour was verified for all three sensors within the same execution cycle.

# 8. Identified limitations

- No physical sensors were available during the sprint; validation was carried out entirely in a simulation environment.

- Ambient humidity measurement could not be implemented due to the absence of a compatible component in the simulation environment.

- The timestamp is relative to device start-up and does not constitute an absolute date and time.

- Soil moisture thresholds correspond to a preliminary calibration and require experimental adjustment with real substrate.

- An intermediate anomaly was observed in the soil moisture acquisition path during testing; it was resolved and re-validated within this same sprint, as documented in section 6.2.

# 9. Work planned for subsequent stages

- Repeat the acquisition tests on real physical sensors and contrast the results with those obtained in simulation.

- Incorporate ambient humidity measurement by means of a DHT22 sensor or equivalent.

- Replace the relative timestamp with an absolute one provided by an RTC module or by the server.

- Migrate the acquisition node to a microcontroller with network connectivity, in accordance with the architecture defined in the project document.

- Implement transmission of the records to the cloud server and their historical storage.

- Register sensor and area identifiers in the database, in accordance with the scheme documented in section 4.

# Annex A. Prototype source code

The code implemented in the acquisition node is included below, corresponding to the version used during this sprint.

// C++ code

// Lectura de 2 sensores de humedad de suelo + 1 sensor de temperatura (TMP36)

// Sprint 1 - SmartGreenAI: Cilantro Crop

// Cumple US-03 (temperatura, parcial), US-04 (humedad de suelo) y US-05 (trazabilidad por ID)

// ---- Identificación de invernadero / área (US-05) ----

const char* GREENHOUSE_ID = "GH-01";

const char* AREA_ID       = "AREA-1";

// ---- Identificación única de cada sensor (US-05) ----

const char* SOIL_SENSOR_ID_1 = "SM-01";   // Soil Moisture sensor 1

const char* SOIL_SENSOR_ID_2 = "SM-02";   // Soil Moisture sensor 2

const char* TEMP_SENSOR_ID   = "TMP-01";  // TMP36 temperature sensor

// ---- Pines ----

const int SOIL_POWER_PIN = A0;  // Alimentación compartida de ambos sensores de humedad

const int SOIL1_SIG_PIN  = A1;  // Señal del sensor de humedad 1

const int TEMP_PIN       = A2;  // Vout del TMP36

const int SOIL2_SIG_PIN  = A3;  // Señal del sensor de humedad 2

int moisture1 = 0;

int moisture2 = 0;

float voltage = 0;

float temperatureC = 0;

// Envía una lectura en formato trazable: invernadero, área, sensor, tipo, valor, unidad, timestamp(ms)

void printReading(const char* sensorId, const char* type, float value, const char* unit) {

Serial.print("{");

Serial.print("\"greenhouse_id\":\""); Serial.print(GREENHOUSE_ID); Serial.print("\",");

Serial.print("\"area_id\":\"");       Serial.print(AREA_ID);       Serial.print("\",");

Serial.print("\"sensor_id\":\"");     Serial.print(sensorId);      Serial.print("\",");

Serial.print("\"type\":\"");          Serial.print(type);          Serial.print("\",");

Serial.print("\"value\":");           Serial.print(value);         Serial.print(",");

Serial.print("\"unit\":\"");          Serial.print(unit);          Serial.print("\",");

Serial.print("\"timestamp_ms\":");    Serial.print(millis());

Serial.println("}");

}

void setup()

{

pinMode(SOIL_POWER_PIN, OUTPUT);

pinMode(SOIL1_SIG_PIN, INPUT);

pinMode(SOIL2_SIG_PIN, INPUT);

pinMode(TEMP_PIN, INPUT);

Serial.begin(9600);

}

void loop()

{

// ---- Sensores de humedad (alimentación compartida) ----

digitalWrite(SOIL_POWER_PIN, HIGH);

delay(10);

moisture1 = analogRead(SOIL1_SIG_PIN);

moisture2 = analogRead(SOIL2_SIG_PIN);

digitalWrite(SOIL_POWER_PIN, LOW);   // Apaga los sensores para evitar corrosión

// ---- Sensor de temperatura TMP36 ----

int tempLectura = analogRead(TEMP_PIN);

voltage = tempLectura * (5.0 / 1023.0);

temperatureC = (voltage - 0.5) * 100.0;

// ---- Mostrar datos por el Monitor Serial (formato trazable, US-05) ----

printReading(SOIL_SENSOR_ID_1, "soil_moisture", moisture1, "raw");

printReading(SOIL_SENSOR_ID_2, "soil_moisture", moisture2, "raw");

printReading(TEMP_SENSOR_ID, "temperature", temperatureC, "C");

delay(500);

}